Code for Measuring Distance with HC-SR04:

#define TRIG_PIN D1  // Trigger pin

#define ECHO_PIN D2  // Echo pin

void setup() {

Serial.begin(115200);  // Initialize serial communication

pinMode(TRIG_PIN, OUTPUT);  // Set the trig pin as an output

pinMode(ECHO_PIN, INPUT);   // Set the echo pin as an input

}

void loop() {

long duration, distance;

// Clear the trigPin

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Send a 10us HIGH pulse to trigger the ultrasonic sensor

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read the time it takes for the echo to return

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance in centimeters

distance = (duration * 0.0343) / 2;  // Speed of sound is ~343 m/s or 0.0343 cm/us

// Print the distance to the Serial Monitor

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

// Wait 500 milliseconds before the next reading

delay(500);

}

Code for Measuring Humidity and Temperature with DHT11/DHT22:

#include "DHT.h"

#define DHTPIN D4  // Pin where the data pin of DHT sensor is connected

#define DHTTYPE DHT11  // DHT11 or DHT22 (replace with DHT22 if you're using that)

DHT dht(DHTPIN, DHTTYPE);  // Initialize DHT sensor

void setup() {

Serial.begin(115200);  // Start serial communication

dht.begin();  // Initialize the sensor

}

void loop() {

// Wait a few seconds between measurements

delay(2000);

// Read humidity

float humidity = dht.readHumidity();

// Read temperature as Celsius

float temperature = dht.readTemperature();

// Check if any reads failed and exit early

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Print the humidity and temperature values to Serial Monitor

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.print(" %\t");

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" *C");

}

Buzzer Triggers if Distance is Less Than a Threshold:

#define TRIG_PIN D1

#define ECHO_PIN D2

#define BUZZER_PIN D3

void setup() {

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(115200);

}

void loop() {

long duration, distance;

// Clear the trigPin

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Trigger the sensor

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read the echo time

duration = pulseIn(ECHO_PIN, HIGH);

distance = (duration * 0.0343) / 2;

// Check if the distance is below a certain threshold

if (distance < 10) {  // Trigger buzzer if object is closer than 10 cm

digitalWrite(BUZZER_PIN, HIGH);  // Turn on the buzzer

} else {

digitalWrite(BUZZER_PIN, LOW);  // Turn off the buzzer

}

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(500);

}

Code for Traffic Light:

#define RED_PIN D5

#define YELLOW_PIN D6

#define GREEN_PIN D7

void setup() {

// Initialize the LED pins as output

pinMode(RED_PIN, OUTPUT);

pinMode(YELLOW_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

// Start with the red light on

digitalWrite(RED_PIN, HIGH);

digitalWrite(YELLOW_PIN, LOW);

digitalWrite(GREEN_PIN, LOW);

}

void loop() {

// Red light on for 5 seconds

digitalWrite(RED_PIN, HIGH);

digitalWrite(YELLOW_PIN, LOW);

digitalWrite(GREEN_PIN, LOW);

delay(5000);

// Yellow light on for 2 seconds

digitalWrite(RED_PIN, LOW);

digitalWrite(YELLOW_PIN, HIGH);

digitalWrite(GREEN_PIN, LOW);

delay(2000);

// Green light on for 5 seconds

digitalWrite(RED_PIN, LOW);

digitalWrite(YELLOW_PIN, LOW);

digitalWrite(GREEN_PIN, HIGH);

delay(5000);

}